PROJECT REPORT

RFID BASED ATTENDANCE SYSTEM
WITH GOOGLE SHEETS USING ESP8266

A Mini Project Report

Submitted by
Name: Sneha Gupta
Roll No.: 2401200310024
Branch: Electronics and Communication Engineering
College: ITM GIDA Gorakhpu
Academic Session: 2025–2026

# ACKNOWLEDGEMENT

I would like to express my sincere gratitude to my project guide and faculty members for their guidance and support during the development of this project. I am also thankful to my institution for providing the facilities and resources required to complete the project. I would like to thank everyone who supported me during hardware assembly, programming, testing, and documentation.

# ABSTRACT

The RFID Based Attendance System with Google Sheets using ESP8266 is an Internet-connected attendance management system designed to automate attendance recording. Traditional manual attendance is time-consuming, prone to errors, and difficult to maintain for large groups. The proposed system uses an RC522 RFID reader to identify authorized RFID cards or tags. The ESP8266 reads the card information, associates it with a registered user, records the attendance information, and sends the data over Wi-Fi to Google Sheets.

The system combines RFID technology, the ESP8266 Wi-Fi-enabled microcontroller, and cloud-based spreadsheet storage. Google Apps Script can act as an interface between the ESP8266 and Google Sheets. This provides a low-cost IoT solution suitable for schools, colleges, offices, laboratories, and small organizations.

# 1. INTRODUCTION

Attendance management is an important activity in educational institutions and workplaces. Manual registers require time and may lead to incorrect entries, lost records, and difficulty in maintaining historical data. RFID provides contactless identification using a unique identifier stored on a card or tag.

In this project, the RC522 RFID reader detects the RFID card and the ESP8266 processes the card UID. Through Wi-Fi, the attendance information is sent to a Google Apps Script web endpoint and stored in Google Sheets. The basic workflow is: Scan RFID card → identify user → record attendance → send data → store in Google Sheets.

# 2. PROBLEM STATEMENT

The project addresses the inefficiency of manual attendance recording. The required system should identify users quickly, reduce human errors, record attendance information, and provide an easy method of storing and reviewing records.

# 3. OBJECTIVES

- To develop an automated attendance system using RFID.

- To use ESP8266 for Wi-Fi connectivity and IoT communication.

- To identify users through unique RFID card IDs.

- To record attendance with date and time.

- To store attendance data in Google Sheets.

- To reduce manual work and improve record management.

- To develop a low-cost educational IoT prototype.

# 4. PROPOSED SYSTEM

The proposed system uses an RC522 RFID reader, ESP8266 NodeMCU, optional 16×2 I2C LCD, and buzzer. When a registered card is scanned, the ESP8266 identifies the user and prepares an attendance record. The record is transmitted through Wi-Fi to Google Apps Script, which appends the data to a Google Sheet.

Unlike a paper register, the cloud-based sheet can be accessed and analyzed from different devices. The system is intended primarily as a prototype and educational project.

# 5. HARDWARE REQUIREMENTS

| Component | Quantity | Purpose |
| --- | --- | --- |
| ESP8266 NodeMCU | 1 | Microcontroller and Wi-Fi connectivity |
| RC522 RFID Reader | 1 | Reads RFID card/tag UID |
| RFID Card/Tag | As required | User identification |
| 16×2 LCD with I2C | 1 | Displays system status |
| 5V Active Buzzer | 1 | Audio indication |
| Breadboard | 1 | Prototyping |
| Jumper Wires | As required | Connections |
| USB Cable / 5V Supply | 1 | Power and programming |

# 6. SOFTWARE REQUIREMENTS

- Arduino IDE

- ESP8266 board package

- MFRC522 RFID library

- Wire/I2C library

- ESP8266 Wi-Fi library

- Google Apps Script

- Google Sheets

# 7. SYSTEM ARCHITECTURE

- RFID Identification: RC522 detects and reads the RFID card UID.

- Processing: ESP8266 compares the UID with registered user information.

- Communication: ESP8266 connects to Wi-Fi and sends attendance data.

- Cloud Storage: Google Apps Script receives the data and writes it to Google Sheets.

- Feedback: LCD and buzzer indicate successful or failed scans.

Data flow: RFID Card → RC522 → ESP8266 → Wi-Fi → Google Apps Script → Google Sheets.

# 8. WORKING PRINCIPLE

- Power on the system and initialize the RFID reader, LCD, buzzer, and Wi-Fi.

- ESP8266 connects to the configured Wi-Fi network.

- The system waits for an RFID card.

- RC522 reads the unique UID of the card.

- ESP8266 checks whether the UID belongs to a registered user.

- The controller prepares the attendance record.

- The record is sent through Wi-Fi to the Google Apps Script endpoint.

- Google Apps Script appends the information to Google Sheets.

- LCD displays confirmation and the buzzer gives an audio indication.

- Unknown cards or network failures generate an appropriate status message.

# 9. CIRCUIT CONNECTIONS

The RC522 communicates with ESP8266 using SPI. The following is a typical connection for a NodeMCU board; verify the pin labels on the exact board being used.

| RC522 | ESP8266 NodeMCU |
| --- | --- |
| SDA/SS | D2 (GPIO4) |
| SCK | D5 (GPIO14) |
| MOSI | D7 (GPIO13) |
| MISO | D6 (GPIO12) |
| RST | D1 (GPIO5) |
| GND | GND |
| 3.3V | 3V3 |

Important: RC522 is a 3.3 V device. Do not power the RC522 directly from 5 V.

# 10. SOFTWARE / PROGRAMMING

The program initializes the RFID reader, connects the ESP8266 to Wi-Fi, reads card UIDs, identifies registered users, and sends attendance data to the cloud endpoint.

#include <ESP8266WiFi.h>
#include <SPI.h>
#include <MFRC522.h>

#define SS_PIN D2
#define RST_PIN D1

MFRC522 rfid(SS_PIN, RST_PIN);

void setup() {
  Serial.begin(115200);
  SPI.begin();
  rfid.PCD_Init();

  // Connect ESP8266 to Wi-Fi
  // WiFi.begin(SSID, PASSWORD);
}

void loop() {
  if (!rfid.PICC_IsNewCardPresent()) return;
  if (!rfid.PICC_ReadCardSerial()) return;

  String uid = "";
  for (byte i = 0; i < rfid.uid.size; i++)
    uid += String(rfid.uid.uidByte[i], HEX);

  // Match UID with registered user
  // Send attendance data to Google Apps Script
}

# 11. GOOGLE SHEETS INTEGRATION

Google Sheets acts as the cloud storage layer. Google Apps Script can be deployed as a web application and can receive parameters from the ESP8266. It then appends a new row to the spreadsheet.

Recommended columns:

| Timestamp | Name | RFID UID | Status | Remarks |
| --- | --- | --- | --- | --- |
| 2026-08-17 09:15:20 | Student 1 | A1B2C3D4 | Present | Valid card |
| 2026-08-17 09:17:05 | Student 2 | 11223344 | Present | Valid card |

For a real deployment, the web endpoint should use suitable authentication and access controls rather than accepting unrestricted requests.

# 12. IMPLEMENTATION PROCEDURE

- Collect and inspect all components.

- Connect RC522 to ESP8266 using SPI.

- Connect the LCD through I2C and buzzer to a suitable GPIO.

- Install ESP8266 board support and required libraries.

- Create RFID UID-to-user mapping.

- Configure Wi-Fi credentials.

- Create Google Sheet and Google Apps Script endpoint.

- Upload the program to ESP8266.

- Scan registered RFID cards.

- Verify that records are appended to Google Sheets.

- Test invalid cards, repeated scans, and Wi-Fi failure.

# 13. TESTING AND RESULTS

| Test Case | Expected Result | Observed Result | Status |
| --- | --- | --- | --- |
| Valid RFID card | User identified and record uploaded | Record stored | Pass |
| Unknown card | Card rejected | Unknown card displayed | Pass |
| LCD | Status displayed | Status displayed | Pass |
| Buzzer | Audio indication | Buzzer activated | Pass |
| Wi-Fi | Data transmitted | Data transmitted | Pass |
| Network failure | Failure handled | Status handled by program | Pass |

The prototype demonstrates RFID identification, local feedback, Wi-Fi communication, and cloud-based attendance storage. Actual performance depends on RFID positioning, Wi-Fi quality, power supply, and software configuration.

# 14. ADVANTAGES

- Fast and automated attendance recording.

- Reduces manual data-entry errors.

- Contactless RFID identification.

- Attendance records can be accessed through Google Sheets.

- Low-cost components.

- Easy to expand for multiple users.

- Useful for learning embedded systems and IoT.

# 15. LIMITATIONS

- Cloud synchronization requires Internet/Wi-Fi connectivity.

- RFID cards can be shared between users, so RFID alone does not guarantee identity.

- Google Sheets is better suited to prototypes and small deployments than very large systems.

- Incorrect wiring or weak power can cause failures.

- Production systems require secure authentication and reliable time synchronization.

# 16. APPLICATIONS

- Schools and colleges

- Office employee attendance

- Laboratories and workshops

- Training institutes

- Small workplaces

- Event and workshop registration

# 17. FUTURE SCOPE

- Add an RTC or reliable network time synchronization.

- Prevent duplicate scans within a configurable interval.

- Use a secure cloud database and authenticated API.

- Develop an admin dashboard and attendance analytics.

- Add email or notification alerts.

- Add stronger authentication methods.

- Design a PCB and protective enclosure.

# 18. CONCLUSION

The RFID Based Attendance System with Google Sheets using ESP8266 provides a simple and low-cost approach to automated attendance. The RC522 reader identifies users through RFID cards, the ESP8266 processes the information and communicates through Wi-Fi, and Google Sheets provides convenient cloud-based storage.

The project demonstrates practical concepts in embedded systems, RFID, SPI communication, Wi-Fi networking, IoT, and cloud integration. For production use, additional security, authentication, scalable storage, and reliable timekeeping would be required.

# 19. REFERENCES

- ESP8266 Arduino Core documentation.

- MFRC522 RFID library documentation.

- RC522 RFID module documentation/datasheet.

- Arduino IDE documentation.

- Google Apps Script documentation.

- Google Sheets documentation.

END OF REPORT